Ultra SCSI (FAST-20) CardBus PC Card for PC
CB31U

# Features

- Supports Ultra SCSI rate up to 20MB/sec.

- 32bit DataBus and 33MHz operation .

- Supports Bus-Mastering data transfer.

- Ready for ACPI.

- Low power consumption.

- Supports scatter/gather and tagged queue .

- Linux Ready. Available for high-speed data backup.

- Supports Plug & Play, allows adding and removing SCSI devices without system re-start.

# System Requirements

- Windows95B, 95C, 98, 98SE, Me, 2000, XP

- Linux Kernel 2.2, 2.4

- One Free CardBus PC Card Slot

## Device List

[CD-R/W Drives]

| YAMAHA | CRW4416SX, CRW6416SX, CRW8424S |
| --- | --- |
| HP | CD-Writer Plus 9210e, CD-writer 9600se |
| TEAC | CD-R551S, CD-R56S, CD-R824S |
| Ricoh | MP7040SE |
| Plextor | PX-R820, PX-8220Ti |

Other Ultra-SCSI or SCSI CD-R/W drives.

[Film Scanner]

| Nikon | CoolscanIII LS-30, Super CoolScan 2000 |
| --- | --- |
| Canon | FS2710 |

Other Ultra-SCSI or SCSI Film Scanner.

[Scanner]

| Canon | CanoscanFB620S |
| --- | --- |
| EPSON | Perfection 1200S, Expression 1800, Expression 1600 |

Other Ultra-SCSI or SCSI Scanner

[MO Drive]

| Olympus | 640MO TURBO, 640mo TURBO LE, 640S II |
| --- | --- |
| Fujitsu | SMB-640D2 |

[Other Removable Media]

| Iomega | JazV2000S, Zip250 |
| --- | --- |
| Castlewood | ORB 2.2GB |

[Hard Disk drives]

* Learn updated information at  www.ratocsystems.com/english/

# Specifications

| Bus interface | CardBus (PC Card Standard) |
| --- | --- |
| Bus rate | 132MB/sec |
| SCSI data transfer rates | 20MB/sec with Ultra SCSI devices (synchronous) 10MB/sec with FAST SCSI devices (synchronous) |
| Termination type | Internal active termination provided. |
| Package includes | - CB31 Ultra SCSI CardBus PC Card - Hi-Density 50F to Centronics 50M adapter - Hi-Density 50pin cable (2feet) - CB31U software Floppy Disk - CB31 Driver for Windows95/98/Me - CB31 Driver for Windows2000 - CB31 Driver for Linux - SCSI configuration utility - Format utility User's manual - Software License Agreement |
| Physical characteristics | 54mm (W) x 85.6mm (D) x 5mm (H) (Type II), 25g |
| Operating requirements | CardBus PC Card slot Power Requirement - +3.3V - 130mA(idle), 300mA(MAX), 77mA(Power save mode) Temperature 0C to 55C Relative humidity 20% to 80% (non-condensing) |