CardBus PC Card Adapter
CBS52U

Features

- PCI-CardBus Bridge adapter board.

- Supports both 32-bit CardBus PC Cards and 16-bit PC Cards at the same time.

- Allows CardBus PC Cards and 16-bit PC Cards to be shared by both portable and desktop PC.

- Supports both 3.3V Low-voltage 16-bit PC Cards and 3.3V/5V Dual-voltage 16-bit PC Cards.

- Bundled ISA-IRQ routing driver software routes 16-bit PC Card IRQ to shared PCI INT#.

- Automatic detection and configuration both 16-bit PC Card and CardBus PC Card.

- Supports Plug & Play and Hot-Swappable 16bit PC Cards, CardBus PC cards.

- Supports Wireless LAN 802.11a, 802.11b and Bluetooth PC Cards.

- Add two CardBus slot to 1U Server and share SCSI Tape backup streamers with hot swapping.

- Onboard speaker allows modem PC Cards to enable negotiation sounds.

- ACPI support

- Linux ready!

- One year warranty

# Specifications

| Standard | PCI Local Bus 2.2 PC Card Standard 7.0 ACPI (PCI Power management 1.0) |
| --- | --- |
| System requirements | PC with one free PCI slot WindowsXP, 2000, Me, 98SE Linux 2.2, 2.4(Redhat LINUX 7.3) * ISA-IRQ routing driver must be installed to Windows2000/Me/98SE. WindowsXP already includes this driver. |
| Package includes | - CBS52 CardBus PC Card Adapter PCI board - ISA-IRQ routing driver for Windows 2000/Me/98SE. - User guide document |
| Physical characteristics | CardBus Bridge board 140mm (L) X 98mm (W) |
| Power requirements | 5V/150mA via PCI Bus slot. |
| Supported PC Card | - Flash memory cards(PC Card Adapter required for CF,SmartMedia, MemoryStick,and SD Card), - I/O Cards - Modem PC Cards. - LAN PC Cards, CardBus Cards - SCSI PC Cards, CardBus Cards - IEEE1394/FireWire CardBus Cards - USB 2.0 CardBus Cards - 802.11a Wireless CardBus Cards - 802.11b Wireless PC Cards - Bluetooth PC Cards - Wireless Communication PC Cards - HDD PC Cards (Toshiba 1.8') - IBM Microdrive (PC Card Adapter required) - 16-bit 3.3V Low-Voltage PC Cards - 16-bit 3.3V/5V Dual-Voltage PC cards - 16-bit 5V PC Cards - 32-bit CardBus PC Cards - RATOC CardBus PC card products, 16bit PC card products * Does not support ZV-port . * Does not support old 16-bit PC Cards that have incorrect CIS or are not supported by WindowsXP. |