Serial I/O(RS-232C) CF Card
CF60U

Add Hi-Speed COM port
to your PDA, Palmtop PC



Works as a standard COM port

Features

- Add Hi-Speed COM port to your PDA, Palmtop PC.

- Works on Compaq iPAQ, Dell Axim, Toshiba Pocket PC, Fujitsu Pocket-LOOX, Casio and HP PDAs. (One free CF TypeII slot required.)

- Attach bar code scanners, magnetic stripe card readers, GPS receivers or other RS-232C devices.

- Supports modems, ISDN adaptors, instruments devices and biometric sensors.

- Easy to use, just Plug & Play. Works as a standard COM port on WindowsXP, Windows2000, WindowsMe, Windows98, Windows CE, Pocket PC 2000, Pocket PC 2002 and Handheld PC 2000.

- Bundled VB,VC sample source code and libraries allow you to make your own communication software.

- Ultra low power consumption allows a long battery life.

- Optional (not included) TypeII PC Card adapter allows to fit CF60U into a standard PC Card slots and use with Laptop PC.

System Requirements
One free CF TypeII compatible slot.

OS
- WindowsXP, Windows2000, WindowsMe and Windows98/98SE.
 Pocket PC 2000, Pocket PC 2002, Handheld PC 2000, WindowsCE 2.0 or later.

Certified PDA and USB devices

PDA

Compaq : iPAQ with CF Jacket, PC Card Jacket.

Toshiba : Pocket PC e740

Dell : Axim

Casio : Casiopea

Fujitsu : Pocket Loox

RS-232C device

Analogue Modems, ISDN modems,

Bar code scanners, Magnetic stripe card readers, RF-ID tag readers,

Biometric sensors, Instrumental devices

Specifications

| Product name | Serial I/O(RS-232C) CF Card |
| --- | --- |
| Product Number | CF60U |
| Bus Interface | CF+ and Compact Flash Specification Rev.1.4 compliant |
| Serial port interface | EIA/TIA-574 compliant |
| Serial port Input/Output level | EIA/TIA-232 compatible (Output +/-5V, Input +/-15V to +/-5V) |
| Serial Controller | 16650 compatible UART |
| Maximum Baud Rate | 115,200 bps. |
| Data format | Asynchronous, Start bit 1, Data bit 7/8, Parity bit Even/Odd/None, Stop bit 1/1.5/2. |
| Serial Port Connector | 9 pin D Shell male |
| Physical Dimensions | CF TypeII 42.8(W) x 36.4(L) x 4.9(H) mm 1.69(W) x 1.43(L) x 0.13(H) inch |
| Cable length | 500mm (20 inch) |
| Weights | CF Card : 10g. 0.35oz. Cable : 55g. 1.93oz. |
| Power Consumption | Standby; Typical. 5V Vcc; 1.2mA; 11.8mA. 3.3V Vcc; 2.2mA; 8mA. |
| OS Support | WindowsXP/2000/Me/98, WindowsCE, Pocket PC 2000, Pocket PC 2002, Handheld PC 2000. |
| Certification | VCCI/FCC Part 15, Class B. |
| Warranty | Removable Cable 90 days, Card 5 years. |
| Package includes | - CF60U Serial I/O CF card - Removable cable. - Software CD-ROM < Software CD-ROM contents > -INF file for WindowsMe/98 -[Visual Basic/Visual C] Communication program sample source code. For Windows98/Me/2000/XP, WindowsCE, Pocket PC 2000,Pocket PC 2002, Handheld PC 2000. - User's Guide |

* Covered and licensed by U.S.Patents 4,603,320 4,686,506 4,972.470.