USB2.0 Removable Case kit for 3.5” HDD
U2-MDK1

Allows HDD Exchanging 
without System Shutdown or Restart.

- Allows Large Capacity (250GB) - Plug & Play, Hot-Swappable. - Compatible with ATA133/100/UDMA HDD,ATAPI MO/Zip. - Best for DV Editing.

Features

- This kit includes one docking bay with USB2.0 to UDMA IDE interface module and one drive tray with mounting screws.

- The HDD can work as a hard disk mode besides a removable disk mode.

- The interface module includes RATOC's original circuit and firmware for high-performance USB2.0 to IDE conversion. It supports UDMA mode 4.

- The docking bay includes Hi-reliability 96P DIN connector, low-noise(25db-A) low power(12V/50mA) cooling fan(Air Flow 5.19CFM), and LED indicators for Power and HDD access status.

- The HDD tray includes 80-conductors ATA133/100 cable and convenient pull-out handle.

- Support Maxtor 250GB and 'Big Drive'/48bit LBA.

- Just fits in any PC tower case, external subsystem case and RAID case.

- Easy installation and usage.

- Plug and Play, recognized and mounted by the OS automatically.

- Hot-Swappable, exchangable HDD without System shutdown and restart.

- No Need for ID setting or terminators.

- Fully backward compatible with USB1.1 Host adapter and USB ports

Unlock...

Allows HDD Exchanging.

System Requirements

PC with USB2.0/1.1 port

Windows XP/Me/2000/98SE

|  | ICH4 M/B | USB2.0 PCI Board |
| --- | --- | --- |
| PU | Intel(R) Pentium(R) 4 CPU 1500MHz | Intel(R) Pentium(R) 4 CPU 1500MHz |
| M/B | ASUS P4B533（845E) | FIC VC11 |
| Memory | 256MB | 256MB |
| HDD | IBM Deskstar 120GXP 40GB IC35L040AVVA07-0 | Maxtor 4G160J8 (160GB 5400rpm UltraATA133) |
| OS | WindowsXP | Windows2000 |
| USB2.0 port | M/B Standard(ICH4) with Microsoft driver | REX-PCIU2 |
| Bench Mark Test Software | WinBench 99 | WinBench 99 |

Specifications

| Features | USB2.0 Removable case kit |
| --- | --- |
| Interface | Upstream USB2.0 480Mbps 1 port. Compatible with USB2.0 Spec. Downstream IDE ATA133/100/66/33, ATAPI MO/Zip compatible. |
| Data transfer rate | 480Mbps/12Mbps/1.5Mbps |
| Power Consunption | -DC +12V/80mA (Cooling Fan) through the Power Connector -DC +5V/250mA(Max) (Logic board) through the Power Connector -DC +5V/300mA (Depends on HDD) through the Power Connector -DC +12V/600mA (Depends on HDD) through the Power Connector *Use each HDD Power cable for each U2-MDK1. |
| Physical Dimensions | 8.4 x 5.8 x 1.5 inches(D/W/H). Fits in 5" half-height bay of any PC tower case, external subsystem case and RAID system case. |
| Weights | 490g (excluding Drive) |
| Package includes | -USB2.0 Removable docking bay -HDD tray -Mounting Screw kit -Tray lock key set -Cable for PCIU2U/PCIFU2U -Cable for ICH4 M/B -Driver and Utility software CD-ROM - Mass storage class driver for Windows98SE. - Hard Disk Drive/Removable Media formatting and partitioning software for WindowsMe/98SE. - Physical formatting software. |
| Operating requirements | Temperature from 0 °Cto 55 °C Relative humidity 20-80% (Non-condensing) |

Installation

Fits to any PC tower case, 
external Subsystem & RAID system cases.

| # U2Dock Series Products [USB2.0 Removable Case kit for 3.5" HDD] | ### Option |
| --- | --- |
| External 1 Bay Type U2-DK1 | HDD Tray FR-MR1 |
| * HDD is not included. |  |